本科毕业设计（论文）

基于时空非单调相关可塑性的神经形态突触器件仿真验证

学    院       集成电路学院

专    业     微电子科学与工程

年级班别      2022级（1）班

学    号        3122009664

学生姓名          黎展睿

任课教师          陈雅怡

2026年 4月

摘要

随着人工智能与物联网技术的快速发展，传统冯·诺依曼架构逐渐暴露出存储墙和高功耗等问题，难以同时满足高并行、低延迟和低能耗的信息处理需求。神经形态计算通过在硬件层面融合存储与计算、模拟生物神经系统的事件驱动和并行处理机制，被认为是突破上述瓶颈的重要方向。作为神经形态系统的重要基础单元，能够实现可塑性调控的突触器件因此成为当前研究热点。

现有研究大多集中于时间相关可塑性，而对空间相关可塑性的探索仍较有限；已有空间可塑性研究通常局限于微米尺度，并多表现为随距离增加而单调衰减的响应规律。与此同时，空间可塑性的物理机制尚缺乏充分解释，尤其是在突触输入物理位置如何调控器件权重更新方面，仍缺少系统性的建模与验证。针对这些不足，本文以多侧栅树突突触晶体管为对象，尝试在更大空间尺度上研究时空耦合下的非单调相关可塑性，并从器件、模型和应用三个层面建立相互印证的分析框架。

在模型与方法方面，本文结合COMSOL和Silvaco TCAD开展联合仿真。在本研究中，我们展示了一种晶体管结构的人工树突（TSAD），在厘米尺度上实现了空间非单调突触效能。观察到的兴奋性突触后电流（EPSC）随距离呈非单调趋势，超越了传统单调离子扩散的限制。结合多物理场模拟与复阻抗分析，我们揭示这一现象来源于离子迁移效率与亚饱和离子环境中有限离子可用性之间的动力学竞争。结合仿真与实测，利用这些内在动力学，我们实现了一种TSAD-RC，对28类复杂时间信号的识别准确率达到100%，阐明了宏观尺度非单调时空整合的物理本质。

关键词：时空非单调可塑性、突触晶体管、多侧栅树突结构、离子动力学分析、器件仿真、算法训练

Abstract

With the rapid advancement of artificial intelligence and the Internet of Things, the traditional von Neumann architecture has increasingly revealed shortcomings such as the memory wall and high-power consumption, making it difficult to simultaneously satisfy the demands of highly parallel, low-latency, and energy-efficient information processing. Neuromorphic computing, which integrates storage and computation at the hardware level and emulates the event-driven and parallel-processing mechanisms of biological neural systems, is widely regarded as a promising route to overcoming these bottlenecks. Accordingly, synaptic devices capable of plasticity modulation have become a major focus of current research as fundamental building blocks of neuromorphic systems.

Existing research has largely concentrated on temporal plasticity, whereas spatial plasticity has received relatively limited attention. Reported studies of spatial plasticity are typically restricted to the micrometer scale and generally show a monotonic decay in response with increasing distance. Meanwhile, the physical mechanisms underlying spatial plasticity remain insufficiently understood. In particular, systematic modeling and validation are still lacking with respect to how the physical locations of synaptic inputs regulate device weight updates. To address these issues, this paper investigates spatiotemporally coupled non-monotonic correlated plasticity over a larger spatial scale using a multi-side-gate dendritic synaptic transistor as the research platform, and establishes a mutually corroborative analytical framework spanning device, model, and application.

For modeling and methodology, joint simulations were carried out using COMSOL and Silvaco TCAD. In this work, a transistor-structured artificial dendrite (TSAD) is demonstrated to achieve spatially non-monotonic synaptic efficacy at the centimeter scale. The observed excitatory postsynaptic current (EPSC) exhibits a non-monotonic dependence on distance, surpassing the limitation of conventional monotonic ion diffusion. Combined multiphysics simulations and complex impedance analysis reveal that this phenomenon arises from the dynamic competition between ion migration efficiency and the limited availability of ions in a subsaturated ionic environment. By integrating simulation and experimental results and leveraging these intrinsic dynamics, a TSAD-RC is realized, achieving 100% recognition accuracy for 28 classes of complex temporal signals and thereby elucidating the physical basis of macroscale non-monotonic spatiotemporal integration.

Keywords: spatiotemporal non-monotonic correlated plasticity; synaptic transistor; multi-side-gate dendritic structure; ion dynamics analysis; device simulation; algorithm training

目录

1	绪论	1

1.1	研究背景与意义	1

1.1.1	冯·诺依曼架构瓶颈与神经形态计算的发展需求	1

1.1.2	突触晶体管在神经形态系统中的作用与研究价值	1

1.1.3	时空相关可塑性研究的理论意义与应用意义	2

1.2	国内外研究现状	3

1.2.1	突触晶体管及侧栅/多栅结构的研究进展	3

1.2.2	栅距对 EPSC、PPF 等突触行为影响的研究进展	3

1.2.3	时空信息整合与储备池计算相关研究进展	3

1.2.4	现有研究的局限性与本文研究切入点	4

1.3	本文研究内容与创新点	3

1.4	论文结构与安排	3

参考文献	4

致谢	5

绪论

研究背景与意义

冯·诺依曼架构瓶颈与神经形态计算的发展需求

随着人工智能和物联网技术的快速发展，图像识别、模式分类和时序信号处理等任务对计算系统的并行性、实时性和能效提出了更高要求。传统冯·诺依曼架构中存储单元与计算单元相互分离，数据需要在二者之间频繁搬运，容易引发“存储墙”问题，并带来延迟增加、效率下降和能耗上升等限制，难以适应神经网络规模持续扩大和信息处理复杂度不断提升的需求。尤其在高并行、事件驱动的智能任务中，传统架构的适配性不足愈发明显，已难以充分满足新型智能硬件对高效感知、计算与存储协同的要求。为突破这一瓶颈，研究者借鉴生物神经系统中神经元与突触协同工作的机制，提出了神经形态计算。该范式强调在硬件层面实现存储与计算融合，具备并行处理、事件驱动和自适应学习等特点，被认为是实现高效智能信息处理的重要路径，也为新型类脑器件与系统结构研究提供了明确方向。

突触晶体管在神经形态系统中的作用与研究价值

在神经形态系统中，突触负责信息传递、权值调节以及学习记忆，是实现类脑计算的关键单元。因此，能够模拟突触行为的器件是神经形态硬件实现的重要基础。突触晶体管兼具晶体管的电学调控特性与突触功能模拟能力，可通过栅极调控和脉冲刺激实现沟道电导连续变化，进而表征兴奋性突触后电流、成对脉冲易化以及短时、长时可塑性等典型行为。与传统器件相比，突触晶体管在结构设计、参数调控和多端输入响应方面更具灵活性，既可承担类突触权值存储与更新功能，也有助于实现多输入信息整合和神经网络硬件映射。特别是在多侧栅结构中，其对多路信号的协同调制能力更为突出，因此成为连接器件特性、突触行为与系统应用的重要载体，在神经形态器件研究中具有突出的理论与工程价值，也为后续开展复杂时空调制研究提供了器件基础。

时空相关可塑性研究的理论意义与应用意义

生物神经系统对信息的处理具有明显的时间相关性与空间相关性，突触可塑性不仅取决于刺激强度，还与输入的先后顺序、位置分布及多输入协同作用密切相关。因此，从时空相关角度研究突触可塑性，有助于更真实地揭示神经信息编码与学习机制，并为类脑模型构建提供依据。对突触晶体管而言，时空相关可塑性研究能够反映器件在复杂刺激条件下的动态响应和多输入整合能力，并进一步评估其在特征提取、非线性映射和动态状态表征中的应用潜力。这些特征直接关系到系统对时空信息的利用效率，对于图像识别、模式分类、时序信号处理以及储备池计算等任务具有重要意义，也为后续开展多侧栅结构器件的时空非单调调制行为的研究、机制分析与仿真验证奠定了基础。与此同时，该研究也有助于推动器件层特性向系统级功能实现的有效转化。

国内外研究现状

突触晶体管及侧栅/多栅结构的研究进展

突触晶体管是通过栅控实现沟道电导连续调节的三端器件，从早期单晶体管硅突触到后续电解质栅控有机晶体管，相关研究已证明这一路线能够稳定模拟权值存储与突触可塑性调节过程[1-2]。随着晶体管型人工突触研究的系统展开，已有综述普遍认为三端结构在读写分离、权值线性调控和低功耗集成方面较两端忆阻器更具优势，因此成为神经形态器件的重要分支[3-4]。在此基础上，侧栅/多栅结构进一步把多个面内输入端引入同一沟道区域，使器件能够表征来自不同空间位置的刺激并模拟树突式输入整合，这一点已在多栅有机神经元晶体管和离子凝胶侧栅石墨烯突触晶体管中得到验证[5-6]。近年的二维铁电晶体管和器件综述研究又表明，晶体管型人工突触正由单纯行为演示转向高精度、多功能和更强结构可设计性的方向发展，但其核心仍在于如何把器件结构优势转化为更有效的时空信息处理能力[7-8]。

栅距对 EPSC、PPF 等突触行为影响的研究进展

在多栅或侧栅突触晶体管研究中，栅极与沟道之间的空间距离已被证明会显著影响兴奋性突触后电流（EPSC）峰值、衰减过程和成对脉冲易化（PPF）等短时动态行为，因此栅距实际上是耦合电场分布与离子迁移过程的关键结构参数[9]。已有研究发现，不同面内栅—沟道距离会造成明显不同的 EPSC 与 PPF 响应，说明空间位置本身已经参与突触权重的动态编码[5]。另有研究在侧向离子栅控石墨烯突触晶体管中观察到器件响应可由成对脉冲易化逐步转向成对脉冲抑制，表明有效栅控长度变化不仅会改变响应强度，还可能改变器件对脉冲序列的过滤方式[10]。还有研究表明，多栅树突结构可以通过不同空间输入形成差异化时空整合响应，说明结构尺度与输入位置共同决定了器件的加和、逻辑和突触行为表达[11]。然而，现有文献整体上仍更重视功能演示而非参数规律归纳，对于栅距、局域电容、离子扩散和瞬态权重更新之间的定量对应关系，相关综述和研究总结仍显不足[3,8]。

时空信息整合与储备池计算相关研究进展

随着多侧栅和树突式结构的发展，突触晶体管的研究目标已由单一突触功能模拟扩展到时空信息整合、序列判别和类脑计算任务映射[9]。除电信号外，侧栅石墨烯突触晶体管还可以引入光刺激辅助记忆与联想学习，这说明多端晶体管平台具备把多模态输入纳入时空编码的潜力[6]。进一步地，多端离子调控晶体管已被用于构建具有宽时间动态范围的硬件储备池，表明器件内生的短时记忆和非线性可以直接承担时序特征展开的功能[12]。另有研究将晶体管结构人工树突、可交换源漏栅结构和厘米级空间扩展结合起来，实现了面向时空相关任务的器件验证，说明人工树突器件在复杂输入处理方面具有进一步拓展的可能[13]。总体而言，现有这类研究更多聚焦于时空整合能力展示、联想记忆或储备池任务验证，为多栅器件的应用探索提供了基础，但对时空耦合条件下突触响应规律本身的讨论仍相对有限，特别是对非单调时空可塑性的专门分析仍不充分[7,14]。

现有研究的局限性与本文研究切入点

总体来看，现有突触晶体管研究在单独时间维度的可塑性模拟方面已较为成熟，围绕短时/长时可塑性、突触权值调节及典型时间响应行为的器件实现与性能表征已经较为充分[1-2]。相比之下，面向时空信号处理的研究虽然已借助多栅、侧栅和树突式结构展示出多输入整合与序列处理能力，但现有多栅器件报道中的可塑性响应大多仍表现为随空间位置、栅距或刺激条件变化而单调增强或单调衰减，对非单调时空可塑性器件的系统研究仍明显不足[4,14]。这意味着相关领域在能够直接反映复杂时空耦合特征的非单调响应器件方面仍存在一定空缺[10-13]。因此，本文的切入点将重点放在多侧栅树突突触晶体管的时空信号处理能力上，着重通过仿真数据确定器件在不同栅距和时间关联条件下的非单调时空可塑性规律，再结合器件制备与实验测试结果对仿真结论进行佐证，从而突出本器件以非单调时空可塑性为核心特征的研究重点。

本文研究内容与创新点

本文围绕多侧栅树突突触晶体管的非单调时空可塑性展开研究，研究内容按步骤编号如下：（1）基于 Silvaco TCAD 建立器件等效电学模型，开展电气仿真，获取 EPSC、PPF 等突触响应与权值调制规律，为后续机制分析提供电学依据；（2）基于 COMSOL 多物理场仿真开展离子动力学分析，研究不同栅距与时间关联条件下的离子迁移、电势分布与等效电容变化，判断并解释非单调时空可塑性的产生条件；（3）完成器件制备并在探针台开展电学测试，获得实验数据与仿真结果对照验证，必要时以时序识别等任务作为辅助应用展示。

本文的创新点主要体现在以下三个方面：一是针对现有研究多停留在单调时空响应、空间尺度偏小的问题，提出面向时空耦合输入的多侧栅树突突触晶体管研究方案，将研究尺度拓展到更大栅距/空间范围并聚焦非单调时空可塑性；二是针对时空相关响应机理解释不足的问题，联合 Silvaco 电学仿真与 COMSOL 离子动力学仿真，揭示离子迁移效率与离子可用性竞争导致的非单调响应机制，实现“机理—模型”层面的明确化；三是针对缺少系统验证的问题，通过器件制备与探针台测试对仿真结论进行实验佐证，并结合 manuscript 中提出的 TSAD-RC 时序识别验证作为辅助说明，体现器件在时空信息处理中的应用潜力而不喧宾夺主。

论文结构与安排

参考文献

[1] DIORIO C, HASLER P, MINCH B A, MEAD C A. A single-transistor silicon synapse[J]. IEEE Transactions on Electron Devices, 1996, 43(11): 1972-1980.

[2] KONG L A, SUN J, QIAN C, et al. Long-term synaptic plasticity simulated in ionic liquid/polymer hybrid electrolyte gated organic transistors[J]. Organic Electronics, 2017, 47: 126-132.

[3] DAI S, ZHAO Y, WANG Y, et al. Recent Advances in Transistor-Based Artificial Synapses[J]. Advanced Functional Materials, 2019, 29: 1903700.

[4] WAN Q, ZHU L. 神经形态晶体管研究进展[J]. 微纳电子与智能制造, 2019.

[5] QIAN C, KONG L A, YANG J, et al. Multi-Gate Organic Neuron Transistors for Spatiotemporal Information Processing[J]. Applied Physics Letters, 2017, 110: 083302.

[6] LIU S, HE X, SU J, et al. A Light-Stimulus Flexible Synaptic Transistor Based on Ion-Gel Side-Gated Graphene for Neuromorphic Computing[J]. Advanced Photonics Research, 2022, 3: 2200174.

[7] WANG Z, ZHOU X, LIU X, et al. Van der Waals ferroelectric transistors: the all-round artificial synapses for high-precision neuromorphic computing[J]. Chip, 2023, 2: 100044.

[8] 朱赛克, 赵毅. 面向神经形态系统的突触器件及芯片综述与展望[J]. 功能材料与器件学报, 2024.

[9] HE Y, NIE S, LIU R, et al. Spatiotemporal Information Processing Emulated by Multiterminal Neuro-Transistor Networks[J]. Advanced Materials, 2019, 31: 1900903.

[10] HE X, WANG K, WEI D, et al. Lateral ionic-gated graphene synaptic transistor with transition from paired-pulse facilitation to depression for filtering and image recognition[J]. Carbon, 2024, 226: 119161.

[11] HUANG Y J, WANG W S, HUANG X, et al. Oxide dendrite transistors gated with polyvinyl alcohol/chitosan hybrid electrolyte for spatiotemporal integration[J]. Journal of Alloys and Compounds, 2025, 1010: 177938.

[12] LIU K, LI J, LI F, et al. A multi-terminal ion-controlled transistor with multifunctionality and wide temporal dynamics for reservoir computing[J]. Nano Research, 2024, 17(5): 4444-4453.

[13] NI Y, ZHANG Y, LIN J, et al. Transistor-Structured Artificial Dendrites for Spatiotemporally Correlated Reservoir Computing[J]. IEEE Electron Device Letters, 2025, 46(10): 1881-1884.

[14] SUN B, GUO T, ZHOU G, et al. Synaptic devices based neuromorphic computing applications in artificial intelligence[J]. Materials Today Physics, 2021, 18: 100393.

致谢